| Crane Company: |  |  |  |  | Crane # |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  |  |  | Lifting Load For |  |  |  | MPC |  |  | Contractor (Name) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prepared by: |  |  |  |  |  |  |  |  |  |  |  |  |  | Time: |  |  |  |  |  |  |  |  |  | Unit / Area: |  |  |  |  |  |  |  |  | Equipment #: |  |  |  |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Permit #: |  |  |  |  |  |  |  |  |  |  |  |  |
| Lifting Capacity for Crane’s Configuration (Specify Weight) |  |  |  |  |  |  |  |  |  |  |  |  | 75% of Capacity |  |  |  |  |  |  |  |  |  |  | Current Wind Speed: |  |  |  |  |  |  |  | Max. Allowed Working Wind Speed: |  |  |  |  |
| Number of Lifts: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Review for Winds over 20 MPH |  |  |  |  |  |  |  | Approved By: |  |  |  |  |
| Lift Description/Rigging Plan: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pre-Job Preparation |  |  |  |  |  |  |  |  | Authorization to STOP Work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Complete the JSA 2. Complete / Obtain permit (if applicable) 3. Review JSA with entire crew |  |  |  |  |  |  |  |  | As an employee or contractor of MPC, YOU have the authorization, without fear of reprimand or retaliation, to immediately STOP any work activity that presents a danger to you, a co-worker, or the public; get involved, question and assist in rectifying any situation that is identified as not being in compliance with safety and health policies; to report any unsafe conditions or acts to MPC supervision and to question any activity that you believe presents a risk to the safety and health of personnel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOOK at the Task |  |  |  |  |  |  |  |  |  |  |  | ANALYZE Potential Hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MITIGATE Hazards |  |  |  |  |  |  |  |  |
| What are the steps for this lift? |  |  |  |  |  |  |  |  |  |  |  | What are the potential hazards I could encounter? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | How can I eliminate or control the hazards? |  |  |  |  |  |  |  |  |
| Crew Signatures of Acknowledgement By signing this section, I have reviewed and understand the Job Task, Potential Hazards, and how to Mitigate the Hazards. I understand the assigned responsibilities. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Print Name: (One person may perform more than one position/task as appropriate) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Signature: |  |  |  |  |  |  |  |  |  |  |  |
| 1.Lift Director |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.Crane Operator |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.Signal Person |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.Rigger |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. Spotter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pre-Lift Checklist To Be Completed by Lift Director |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lifting Crew: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes/NA |  |  |  |  |  |  | Rigging: |  |  |  |  |  |  |  |  | Yes/NA |  |  |
| Qualifications and Training Requirements are current |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rigging has been inspected prior to lifting |  |  |  |  |  |  |  |  |  |  |  |
| Lifting crew roles and responsibilities are understood |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rigging tags/labels are attached and legible |  |  |  |  |  |  |  |  |  |  |  |
| Communication plan established between lifting crew |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rigging is within rated capacity |  |  |  |  |  |  |  |  |  |  |  |
| Crane: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rigging is protected from sharp edges or abrasion |  |  |  |  |  |  |  |  |  |  |  |
| Daily crane inspection has been completed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Taglines are attached where applicable |  |  |  |  |  |  |  |  |  |  |  |
| Post assembly checklist has been completed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lifting Load: |  |  |  |  |  |  |  |  |  |  |  |
| Crane safety devices have been tested |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wind speed verified and weather conditions acceptable |  |  |  |  |  |  |  |  |  |  |  |
| Is crane within +/- 1% level |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Load weight is clearly identified for each lift |  |  |  |  |  |  |  |  |  |  |  |
| Crane matting are adequate for conditions and load |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Is the load < 75% of crane capacity |  |  |  |  |  |  |  |  |  |  |  |
| All underground piping/hazards have been identified |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Load structural integrity is sound |  |  |  |  |  |  |  |  |  |  |  |
| Tail swing has been verified and barricaded (as required) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lift points integrity is sound (pad eye, trunnions, etc.) |  |  |  |  |  |  |  |  |  |  |  |
| Will any part of the equipment, load line or load, while operating up to the equipment’s maximum working radius in the defined work zone, get closer than 20 feet to a power line (Horizontal or Vertical) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Load is rigged level and stable |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Load clearance concerns have been addressed |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Is the load a blind lift |  |  |  |  |  |  |  |  |  |  |  |
| Electrical Hazard Analysis completed and attached |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Are whistles/horns available for the lift |  |  |  |  |  |  |  |  |  |  |  |
| Will the lift be over or potentially over equipment and piping containing greater than 5 % HF based on crane radius, the lift shall require a HF Alky Consequence Analysis approval by owner-operator leadership. Consider Crane Radius! |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Has the Pick and Landing Zone for the load been barricaded |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Has the swing path of the load been determined |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Will work activities in the swing path need to be stopped |  |  |  |  |  |  |  |  |  |  |  |
| HF Alky Consequence Analysis completed and attached |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Have work areas affected by the lift been cleared of non-essential personnel |  |  |  |  |  |  |  |  |  |  |  |
| Post Task Review Identify any unforeseen hazards or incidents encountered while performing the task and discuss the corrective actions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

ATTENTION:  Printed copies should be used with caution.  The user of this document must ensure the current approved version of the document is being used.